MCOM101RTUCHMI

Human Machine Interface

RS232/RS485 MODEM

Features:

- 10’’ Touch screen

- In-Built symbol library

- Address Mapping

- Update program by USB flash

- Alarm, Calendar, Real time clock

- Muli-Serial communication

- Ethernet Communication

- QR code generator

- User / Recipe management

- Script function

| Specifications |  | MCOM101RTUCHMI |  |
| --- | --- | --- | --- |
| Display specification | Type | 10’’ Touch screen |  |
|  | Resolution | 1024 X 600 |  |
|  | Display size | 10” Diagonal |  |
|  | Backlight | LED |  |
|  | Span life | 50000 hrs |  |
|  | LCD Type | High precision four-wire resistive |  |
|  | Brightness | 250 cd/m2 |  |
| Functional specification | Programming | Windows based software for HMI |  |
|  | Processor | Cortex A-7 1GHz |  |
|  | OS | Linux 4.9.11 QT based |  |
|  | Memory | 128M Flash + 64MB RAM |  |
|  | USB | USB2.0 Host + USB2.0 Client |  |
|  | USB slave | Type B, program download/upload only |  |
|  | Serial interface | COM1: RS232, COM2:RS485/422, COM3:RS485 |  |
|  | Ethernet Communication | Yes |  |
|  | DB9 | Male(COM1) |  |
|  | Real-time clock | Yes |  |
| Mechanical specification | Dimensions(WxHxD) MM | 273 x 211 X 43 |  |
|  | Mounting | Panel Mounting |  |
|  | Weight | 1.052Kg |  |
|  | Ingress protection | IP65 (Front panel) |  |
|  | Cover material | PC+ABS plastics |  |
| Electrical specification | Power supply voltage | 24V DC (DC 12∿24V) |  |
|  | Rated power | < 15W |  |
| Environmental specification | Temperature | Operating: -10∿60 °C | Storage: -20∿70 °C |
|  | Humidity | 10∿90% RH (Non-condensing) |  |
|  | Seismic performance | 10∿25Hz (2G/30min in x,y,z directions) |  |

Dimension

All Dimensions are in mm

43

273

Panel cutout cutout

10" HMI

8

Panel cutout

211

DB9 Pin Configuration

| Interface | PIN | PIN |
| --- | --- | --- |
| COM1 | 1 |  |
|  | 2 | RS232 RXD |
|  | 3 | RS232 TXD |
|  | 5 | GND |
|  | 6 |  |

| Interface | PIN | PIN |
| --- | --- | --- |
| COM2 | 1 |  |
|  | 2 |  |
|  | 3 |  |
|  | 7 | RS485 A+ |
|  | 8 | RS485 B - |

| Interface | PIN | PIN |
| --- | --- | --- |
| COM3 | 1 |  |
|  | 2 |  |
|  | 3 |  |
|  | 7 | RS485 A+ |
|  | 8 | RS485 B - |

MODERN COMMUNICATION TECHNOLOGY Sr No. 23, Hissa No. 3/1, Behind Wadekar Industries, Narhe, Pune-411041 MOB.+91 9405134829, 7769827105, 9595383766

Web: -www.mcomtechnology.com