MCOM-PDT

PRESSURE SENSOR WITH DISPLAY

MCOM-PDT series pressure transmitter incorporates a media isolated silicon pressure sensor with range from 0 bar to 10 bar. For OEM that need consistent high levels of performance, reliability and stability the PDT series unit series units offer all 316L stainless steel wetted parts at an unbeatable price performance ratio. A wide choice of output signal types in addition to various electrical connection and pressure port types means the unit is suitable for most applications without modification

MCOM-PDT

PRESSURE SENSOR WITH DISPAY

RS232/RS485 MODEM

APPLICATION

- Water Pressure Measurement

- Liquid and Gas Pressure Measurement

- Hydraulic Systems

- Engine Fuel Pressure Measurement

- Refrigerator and Air compressor

- Autoclaves and ETO Sterlizers

- sewage treatment, water supply, petroleum chemical and pharmaceutical industry

FEATURES

- Gauge, Absolute and Vacuum Pressure Measurement

- Measuring Ranges From 0  to 10 bar (other ranges on request)

- Process Temperature from -40 to +125°C

- Wide Range of pressure Ports and Electrical Connectors

- Housing and Diaphragm made of Premium Stainless Steel

- Customized OEM Version Available

PERFORMANCE

- Accuracy                          ≤ ±0.25% BFSL

- Stability                            ≤ ±0.25% FS

- Insulation Resistance >100MΩ/500VDC

- Over Range Protection 300%F.S.( ≤70kPa) , 200%F.S.(<10Mpa)

- Temperature Drift           <+1.5% FS(-20 to 85°C)

- Response Time                 ≤150ms (up to 90%F.S.)

- Reverse Polarity Protection  Yes

THERMAL

Temperature

- Ambient Temperature      -20°C to 85°C

- Medium Temperature        -40°C to 100°C

- Storage Temperature        -40°C to 110°C

Thermal Limits

- Compensated Range           0°C to 70°C

- TC Zero                             ≤ ±1.5% FS

- TC Span                            ≤ ±1.5% FS

ENVIRONMENTAL

- EMI/RFI Protection             Yes (Electromagnetic Compatibility Directive 2014/30/EU)

- IP Rating                                 IP67

- Vibration                                20g (25∿2000HZ) Sine Curve; IEC 60068-2-6

- 200g/1ms; IEC 60068-2-27

ELECTRICAL

- Output           4-20mA/HART                  1-5V, 1-6V, 0-4V (3 wire)                        0-10V (3 wire)

- Excitation      10-30VDC                     10-30VDC                                                  15-30VDC

WIRING GUIDE

| Output | Pin 1 | Pin 2 | Pin 3 | Pin 4 |
| --- | --- | --- | --- | --- |
| 4-20mA (2-wire) | +V | -V | N/A | N/A |
| Voltage (3-wire) | +V | Ground | Signal | N/.A |
| mV (4-wire) | Signal | Ground | +V | -Signal |

MODERN COMMUNICATION TECHNOLOGY Sr No. 23, Hissa No. 3/1, Behind Wadekar Industries, Narhe, Pune-411041 MOB.+91 9405134829, 7769827105, 9595383766

Web:-www.mcomtechnology.com